HAMBURG WHEEL TRACKER

HAMBURG WHEEL TRACKER

|  | Name of the equipment: Hamburg Wheel Tracker Make & Model: BO38AM I-Stem Registration ID- 3222450 Category of Instrument Characterization and Testing Types of Analysis / Testing Rutting Test Application: It is suitable for testing in various conditions, including wet and dry samples, and meets international standards such as EN, AASHTO, and BS. Description of Instrument A patented wheel tracking device for testing Hot Mix Asphalt resistance to rutting and moisture sensitivity. |
| --- | --- |

| Booking Details Book through I-STEM: https://www.istem.gov.in/ Slot Booking Link I-STEM Slot Booking link for External User | Booking available for Internal and External Both Requisition form for Internals Externals |
| --- | --- |

Contact Details

| Faculty In-charge: Dr. M. Sivakumar Email ID: sivakumarm@nitc.ac.in Phone number: 0495-2286232 | Technical Staff: Abhiraj A R abhirajar@nitc.ac.in Ajin Das C K ajindasck@nitc.ac.in Chandni P R chandnipr@nitc.ac.in | Department Department of Civil Engineering Office Email ID cedoffice@nitc.ac.in Location Pavement Engineering Laboratory Lab Phone No. 0495-2286243 |
| --- | --- | --- |

Features, Working Principle and Specifications

| Features of the equipment - Simultaneous wet and dry testing - Independent wheel motors - Automatic testing - Real-time monitoring - Touch-screen interface - Data export | Unique features/Measurement capabilities, if any - Innovative wheels roll off Mechanism (patented) - Real-time results plot - Measurement of creep slope, stripping inflection point, and stripping slope - Precise rut depth transducers (25 mm ± 0.1 mm) - Advanced temperature control |
| --- | --- |
| Instrument Technical Description and Major Specifications (This Specifications Limited to Major 5) - Wheel load: 705 N - Wheel speed: 20-30 cycles/minute - Temperature control range: from ambient up to 75°±1 °C - Table travel: 230, 260, 280 mm - Power supply: 220V 50-60Hz | Measurement/Sample specifications: - Sample diameter- 150mm - Sample shape- cylindrical - Number of samples for 1 test- 2 samples - Type of sample- Hot mix / Cold mix |

Type of Sample Required for Analysis / Testing (Quantity, Pre-Preparation, State etc.) Guidelines for Sample Submission – User Instructions

User Charges Rs. (GST Extra)

- Sample quantity: One sample per test - Preparation: Ensure the sample is properly filled into the mould and secured before testing - Submission guidelines: - Fill the mould with the HMA sample, ensuring it is properly compacted. - Ensure the device is set to the correct test parameters. - Start the test and monitor the results. - Minimum No. of Days: Required for Analysis: 1 day (test completion) to 3 days (data analysis and report preparation)

| Internal | External Academic Institutes | National R&D Lab | Industry |
| --- | --- | --- | --- |
| 1500/- per test | 2000/- per test | 2500/- per test | 4500/- per test |